##### SECTION 1: PRODUCT NAME

EFFLORESCENCE REMOVER

SECTION 2: PRODUCT DESCRIPTION

EFFLORESCENCE REMOVER is a proprietary blend of mixed acids and cleaners that is yellowish-green in color, non-petroleum, nonflammable and user friendly.

##### SECTION 3: BASIC USE

EFFLORESCENCE REMOVER is a light to medium-duty efflorescence cleaner. EFFLORESCENCE REMOVER is safer to apply and less harmful to the concrete or masonry being cleaned than muriatic acid, the current low-tech method being used for efflorescence cleaning.

EFFLORESCENCE REMOVER can he used on concrete, masonry, limestone or other surfaces with efflorescence.

EFFLORESCENCE REMOVER is specially developed, from a mixed acids base, for the cleaning of efflorescence, while minimizing potential damage to the surface from which it was cleaned.

##### SECTION 4: INSTALLATION SUGGESTIONS

EFFLORESCENCE REMOVER is ready to use, DO NOT DILUTE

Test on sample of material being cleaned prior to use.

Apply using a medium bristled brush on most areas. A non-atomizing, acid resistant sprayer may also be used. For areas where splashing or drippage could cause damage, apply with a sponge, avoiding any surrounding mortar. Apply uniformly in terms of concentration, quantity and duration of scrubbing action. Immediately wash cleaned areas to remove any salts formed from the spent acid.

##### SECTION 5: PRECAUTIONS

- EFFLORESCENCE REMOVER contains acids and should be treated with care.

- Avoid splattering surfaces that are not intended for treatment. Damage could occur.

- Always use the gentlest possible cleaning method or material before progressing towards more aggressive measures.

- Avoid contact with eyes or skin. Always wear rubber/neoprene gloves and splash eyewear protection. An eyewash facility should be nearby and ready for use.

- READ Material Safety Data Sheet prior to using EFFLORESCENCE REMOVE.

##### SECTION 6: TECHNICAL DATA

- Physical: Liquid

- Color: yellowish-green

- pH: 4-5

- Composition: Mixed acids

- Corrosive Strength: Mild

- Splatter Resistance: Good

- Toxic Fumes: None

- Freeze Temperature: 32'F

- Freeze Harm: None

- Spill Clean-Up: Dilute / Flush with water

- Shelf Life: Indefinite

##### SECTION 9: SOME ADVANTAGES

- A safer alternative to cleaning with harsher acid.

- Effective for all kinds of concrete surfaces

- Invisible anti-slip.

- Gentle on mortar during masonry applications

- No hazardous fumes

- Easy to Use

- Can be reapplied as many times as needed.

- User friendly.

- Environmentally safe.